Outcome #1 - Identify rules for decimal addition, subtraction, multiplication, and division. Perform multi-step caliper math problems with two and three- place decimals.

Module Content Measurement is one of the most essential factors in manufacturing. Machinists use decimals for precision measurements, determined by rulers, micrometers, calipers, calculators, and computers. Students will practice reading blueprint dimensions, measure physical objects accurately, and use basic math to perform multi-step math problems.

Targeted ABE/ASE Content Standards 3.OA.3: Solve multistep word problems posed with whole numbers and having whole number answers using the four operations, including problems in which remainders must be interpreted. Represent these problems using equations with a letter standing for the unknown quantity. Assess the reasonableness of answers using mental computation and estimation strategies including rounding. 3.NBT.2: Read and write multi-digit numbers of whole numbers using base-ten numerals, number names, and expanded form. Compare two multi-digit numbers based on meanings of the digits in each place, using >, =, < symbols to record the results of comparisons. 3.NBT.3: Use place value understanding to round multi-digit whole numbers to any place. 3.NBT.5: Multiply a whole number of up to four digits by a one -digit number, and multiply two two-digit numbers, using strategies based on place value and the properties of operations. Illustrate and explain the calculation by using equations, rectangular arrays, and/or models. 3.NBT.6: Find whole-number quotients and remainders with up to four-digit dividends and one-digit divisors, using strategies based on place value, the properties of operations, and/or the relationship between multiplication and division. Illustrate and explain the calculation using equations, rectangular arrays, and/or area models. 4.NS.2: Fluently multiply multi-digit whole numbers using the standard algorithm. 4.R.FW.1 – Understand and use technology systems. 4.S.CC.4 – Demonstrate active listening skills.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions, and written exchanges about a range of topics, texts, and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Metric Tolerances Introduce students to metric tolerances by explaining that dimensions are given in millimeters, with one- and two- place decimals on blueprints. Blueprints and parts for the automotive industry are usually metric. Every dimension has a tolerance, which is either added to or subtracted from the print dimension. Tolerances are based on the number of decimal places in the dimension and should be listed on the blueprint. The target measurement is not an exact number, but the range and measure of each feature on the blueprint must fall within that range. Modeling Tolerances Explain to students that the goal is to aim for the nominal size of the part you are measuring. - Write on board 88.4 + 0.25. - Explain that the tolerance is 88.65 which is 88.4 + 0.25 (the upper limit) or 88.15 which is 88.4 – 0.25 (the lower limit). - The measurement of the part can be right on the limit, either upper or lower, and be acceptable. - It is good practice to go through the blueprint and calculate the upper and lower limits to each dimension before starting a work assignment. Provide students with Worksheet 1-1: Metric Tolerances, 1- and 2- Place Decimals1 (page 4). Walk through the blueprint, pointing out the dimensions are in millimeters and that 1 place decimals have a tolerance of +0.25 and 2 place decimals have a tolerance of +0.12. Review answers with the students. Activity 2 Calculating Dimensions Blueprints illustrate the requirements for a part to be manufactured, indicating the size and location of each feature. Dimensions are not always specified on a blueprint, and it is up to the machinist to calculate a dimension from the measurements of the other features on the blueprint. The machinist must verify the size and location of each hole to be drilled. Students will practice caliper math with 2-place decimals using a blueprint worksheet, then practice measuring with dial and digital calipers. Part 1 Worksheet 1-2: Caliper Math with 2-Place Decimals1 Provide students with Worksheet 1-2: Caliper Math with 2-Place Decimals (page 5) to all students - With the worksheet projected onto the board, walk students through the dimensions already stated on the blueprint and have them note the following: - Æ is the symbol for diameter. - The location of a hole is always given from the center of the hole, and the center is imaginary. - A caliper cannot grab the center of the hole. - The measurement must be taken from the edge of the part to the edge of the hole. - Add half of the hole’s diameter to this measurement. - This calculation is the distance from the edge to the part to the center of the hole. - Using the blueprint example, model calculating the location and size of the hole. - Using the given diameter of the hole, Æ 4.76 (3 decimal places) calculate the radius of the hole - 4.76/2 = 2.38 - The top left measurement on the block is 36.57 millimeters, which should be the distance from the edge of the block to the center of the hole. - Subtract 2.38 from 36.57 = 34.19 - 34.19 is the measurement from the edge of the block to the edge of the circle. - Have students work in pairs to calculate the actual locations for dimensions 1-10 (page 6), using the hole dimensions and the edge-to-edge distances. - Circulate among the groups, answering questions and checking for accuracy. Review answers with students. Part 2 Practice – Measurement Tools - Provide students with dial or digital calipers, rulers, and examples of blocks with holes. - Model the how to use the calipers. - Have students work in pairs to calculate the diameter and radius of the hole, the distance from the edge of the block to the edge of the hole. - This is an opportunity to work with the students, helping them to practice with equipment. Check for understanding. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 1

Assessment Students will identify and understand the use of tolerances used in manufacturing. Students will demonstrate the proper use of measurement tools (digital and manual calipers, micrometers, and rulers) used in manufacturing. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards for personal review and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #2 - Identify rules for fraction addition, subtraction, multiplication, and division. Read fractional numbers from a ruler and perform multi-step caliper math using fractions.

Module Content Fraction arithmetic, caliper math with fractions, converting fractions to decimals, and using various rulers and scales to measure.

Targeted ABE/ASE Content Standards 3.NF.2: Compare two fractions with different numerators and different denominators (e.g., by creating common denominators or numerators, or by comparing to a benchmark fraction such as ½). Recognize that comparisons are valid only when the two fractions refer to the same whole. Record the results of the comparisons with symbols >, =, or <, and justify the conclusions (e.g., by using a visual fraction). 3.NF.8: Add and subtract fractions with unlike denominations (including mixed numbers) by replacing given fractions with equivalent fractions in such a way as to produce and equivalent sum or difference of fractions with like denominators. For example, 2/3 + 5/4 = 8/12 + 15/12 =23/12. 3.NF.13: Solve real word problems involving multiplication of fractions and mixed numbers (e.g., by using visual fraction models or equations to represent the problem). 3.NF.14: Apply and extend previous understanding s of division to divide unit fractions by whole numbers and whole numbers by unit fractions. 4.RP.1: Understand the concept of a ratio and use ratio language to describe a ratio relationship between two quantities. 4.NS.3: Fluently add, subtract, multiply, and divide multi-digit decimals using the standard algorithm for each operation. 4.NS.10: Apply and extend previous understandings of multiplication to multiply a fraction or whole number by a fraction. 4.EE.17 – Perform operations with numbers expressed in scientific notation, including problems where both decimal and scientific notation are used. 4.S.CC.4 – Demonstrate active listening skills. 4.R.FW.1 – Understand and use technology systems.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions, and written exchanges about a range of topics, texts, and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5 / HI.5.2 - Gather information from multiple print and digital sources ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources Discuss with students the importance of understating what a fraction is: a whole number (or integer) that is divided by another whole number. Fractions need to be converted to decimals for machinists and welders to measure precisely. When converting fractions to decimals, there will be times when it is exact and other times the closest decimal number will need to be determined. All materials can be found in the Math Activities & Resources document. Activity 1 Reading a Scale Provide students with Learn It: Reading a 6” Scale & Worksheet 2-1: Reading 8ths & 16ths of an Inch1 (pages 12&13). - Project onto the board the example of the ruler from page 12. - Have them note and highlight on page 12 the following: - Rulers are called scales & reading a scale is reading fractions. - Metalwork rulers, for example, only use 1/2’s, 1/4’s, 1/16’s, 1/31’s, and 1/64’s. - 1/3’s or 1/5’s are not used. - There are several types of rulers used in metalwork: - 4R scale rulers have 8ths and 16ths on one side, and 32nds and 64ths on the other side. - 16R scale has 32nds and 64ths on one side, and 50ths and 100ths on the other side. - Have students follow along as you walk through reading the scale at the points A, B & C on page 12. - Note that it is an example of a 6-inch ruler, with markings on the top, which is always an odd number and in its lowest terms. - Each of those markings represents a fraction of the ruler. - For marker A: - Count the lines from the end. - A is the third line, so it is reading 3/8 of an inch. - For marker B: - B is at the 4th line, so it is 4/8 of an inch (or 1/2). - B is reading at 2 and 1/2 inches. - The bottom of the ruler is graduated in 1/16ths of an inch. - For marker C: - C is 14 lines past 1 which is 1 and 14/16 of an inch. - You will need to reduce 14/16 which changes the reading to 1 and 7/8s of an inch. - Check for understanding. Activity 2 Reading Rulers - Have students collaborate with a partner to complete Worksheet 2-1: Reading 8ths and 16ths of an Inch1 (page 13). - Remind students that to reduce fractions to their lowest, the top number of the fraction should be odd. - Project the worksheet on the board and have one person from each group volunteer their answers. Review answers with students. Activity 3 How to Count the Lines Odd Provide each student with the worksheet Learn It: How to count the lines odd1 (page 14) - Project page 14 on the board and ask students to locate marker D. - Ask: - Which type of line is it touching? - Is it the 16th or 8th? Or something different? - Explain that the smallest type of line on this ruler is the 16th line. - Have them count from the nearest inch line, counting only the 16th lines and count by odd numbers: 1,3,5 and so on. - Students should find that the D marker is at line 13 and the reading is 4 and 13/16 inches. - Note that when counting scales graduated in 16ths, 32nds, or 64ths, count the lines odd. - Review answers and check for understanding. Activity 4 Reading Scales Provide students with Worksheet 2-2: Reading a Scale with 32nds and 64ths, and Worksheet 2-3: 16R Scale, 50ths and 100ths1 (pages 15 & 16). - Have students collaborate with their partners to determine the scale readings. - Project the rulers on the board and have students volunteer their answers on the board. - Remind them to reduce fractions to lowest terms. - Have students explain to the class how they determined the answer. Check for understanding. Activity 5 Reading Scales – Practice Provide students with various rulers and have them measure objects such as blocks, tables, doorways, windows cabinets, books, etc. This is a chance to work with students, help them practice with equipment, and identify and correct mistakes. Activity 6 Using Decimal Equivalents Provide students with Learn It: Decimal Equivalents Chart, Decimal Equivalents Chart and Worksheet 2-5: Converting Fractions to Decimals on a Print (pages 18, 19, and & 20). Project Lean It: Decimal Equivalents Chart (page 18) diagram on the board. - Discuss with students the following points: - Fractions may not be used on all blueprints. - If a fraction is used, convert to a decimal. - Manually - Calculator - Decimal Equivalent Chart - The chart shows a decimal equivalent to three places, which is used for most fractional dimensions. - Have the students convert the fractions on the drawing with a calculator – then have them check the same fraction on the decimal equivalent chart. - Review the answers with students. Project Worksheet 2-5: Converting Fractions to Decimals on a Print (page 20) on the board. - Have students use the chart to convert the fractions to decimal equivalents. - Check for understanding. Activity 7 Converting Reading Scales – Practice – To Decimals Have the students take their measurements from Activity 5 and convert to decimals using the Decimal Equivalent Chart. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 2

Assessment Students will demonstrate mastery in converting fractions to decimals and accuracy and precision in measuring with rulers. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #3 - Identify and understand dimensions and tolerances. Calculate the limits of decimal and fractional tolerances.

Module Content Calculate limits for dimensions fractions, 2- and 3- place decimals, special tolerances, and limits for 3-4 place dimensions.

Targeted ABE/ASE Content Standards 3.NF.2: Compare two fractions with different numerators and different denominators (e.g., by creating common denominators or numerators, or by comparing to a benchmark fraction such as ½). Recognize that comparisons are valid only when the two fractions refer to the same whole. Record the results of the comparisons with symbols >, =, or <, and justify the conclusions (e.g., by using a visual fraction). 3.NF.8: Add and subtract fractions with unlike denominations (including mixed numbers) by replacing given fractions with equivalent fractions in such a way as to produce and equivalent sum or difference of fractions with like denominators. For example, 2/3 + 5/4 = 8/12 + 15/12 =23/12. 3.NF.13: Solve real word problems involving multiplication of fractions and mixed numbers (e.g., by using visual fraction models or equations to represent the problem). 3.NF.14: Apply and extend previous understanding s of division to divide unit fractions by whole numbers and whole numbers by unit fractions. 4.RP.1: Understand the concept of a ratio and use ratio language to describe a ratio relationship between two quantities. 4.NS.3: Fluently add, subtract, multiply, and divide multi-digit decimals using the standard algorithm for each operation. 4.NS.10: Apply and extend previous understandings of multiplication to multiply a fraction or whole number by a fraction. 4.S.CC.4 – Demonstrate active listening skills. 4.R.FW.1 – Understand and use technology systems.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions and written exchanges about a range of topics, texts and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5 / HI.5.2 - Gather information from multiple print and digital sources ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Definitions: Dimensions vs. Tolerances1 Notes Use the ‘Meet It’ and ‘Learn It’ sections of Unit 3 in the Math Activities & Resources document as additional teaching material for this introductory activity. - Write on the board the following terms and definitions: - Tolerance: how far you can stray from the nominal size of a feature - Dimension: specified nominal size of a feature on the part to be made, expressed as a fraction or decimal. - Ask students: - Are there some dimensions that are more critical than others? - The functional importance of that feature, such as the shelves of a bookcase - The length and width are more critical than the thickness of the shelf - Does everything they make or build come out perfectly? Is it acceptable to say it is good enough? Manufacturing involves making parts to plans that were created by someone else. Not every part made will be exactly as the plan calls for. So those making the plans state the maximum deviation they can tolerate. This is called tolerance, and it is stated as + and a decimal. Write the following example on the board: You inspect five parts that have a 0.312 dimension. You get the following readings: 0.306; 0.310; 0.311; 0.312; 0.307. Ask students which parts are acceptable. Only one is at 0.312, so are the others bad? - Write on the board the tolerance is +0.005. - Now, which parts are acceptable? (0.306 is bad, the rest are acceptable) - What if the tolerance was + 0.010? (all five are good) - All dimensions have a specified tolerance, which allows you to infer the lower and upper control limits for that feature. Activity 2 Fractional and Decimal Dimensions1 Notes Write on the board: ½ .50 .500 Ask students, are these the same or different? (all the same) Discuss with students that dimensions on blueprints can be written three separate ways to communicate the information about tolerance. Tolerances are printed on the blueprint in a block. Unless otherwise specified, tolerances are as follows: 3-Place Decimals; + 0.005. 2-Place Decimals; + 0.010. Fractional; + 1/64. Write these examples on the board and have students calculate the upper and lower tolerances. 0.500 0.50 0.63 0.630 1/2 - Ask students how this could affects the workplace. What should they do when given a blueprint to make a part with several dimensions? Check the first part – make any adjustments to ensure the dimensions are closer to nominal - Which of the dimension tolerances would require closer inspection? 3-Place - Ask what do they need to do if the tolerance is given as a decimal, but the dimension is a fraction? Convert the fraction to a decimal and calculate the upper and lower limits Activity 3 Practice Calculating Upper & Lower Limits - Provide Worksheet 3-1: 2 Place, 3- Place, and Fractional Tolerances (page 28) to all students. - Review with students the HOT TIPS box on the bottom of the worksheet. - Have students collaborate with a partner to calculate the upper and lower tolerance limits for the dimensions. - Review answers with students. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 3

Assessment Students will master converting fractions to decimals, identify critical dimension measurements in manufacturing, and calculate the upper and lower limits for fractional tolerances. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #4 - Identify and calculate tolerances as portion of a dimension, using addition and subtraction.

Module Content Percent is used to indicate a fraction or portion of a part. Allowable burr size will be calculated using tolerance on a dimension.

Targeted ABE/ASE Content Standards 3.OA.3: Solve multistep word problems posed with whole numbers and having whole number answers using the four operations, including problems in which remainders must be interpreted. Represent these problems using equations with a letter standing for the unknown quantity. Assess the reasonableness of answers using mental computation and estimation strategies including rounding. 3.NBT.3: Use place value understanding to round multi-digit whole numbers to any place. 3.NBT.5: Multiply a whole number of up to four digits by a one -digit number, and multiply two two-digit numbers, using strategies based on place value and the properties of operations. Illustrate and explain the calculation by using equations, rectangular arrays, and/or models. 3.NBT.6: Find whole-number quotients and remainders with up to four-digit dividends and one-digit divisors, using strategies based on place value, the properties of operations, and/or the relationship between multiplication and division. Illustrate and explain the calculation using equations, rectangular arrays, and/or area models. 3.NF.8: Add and subtract fractions with unlike denominations (including mixed numbers) by replacing given fractions with equivalent fractions in such a way as to produce and equivalent sum or difference of fractions with like denominators. For example, 2/3 + 5/4 = 8/12 + 15/12 =23/12. 3.NF.13: Solve real word problems involving multiplication of fractions and mixed numbers (e.g., by using visual fraction models or equations to represent the problem). 3.NF.14: Apply and extend previous understanding s of division to divide unit fractions by whole numbers and whole numbers by unit fractions. 4.RP.1: Understand the concept of a ratio and use ratio language to describe a ratio relationship between two quantities. 4.NS.3: Fluently add, subtract, multiply, and divide multi-digit decimals using the standard algorithm for each operation. 4.NS.10: Apply and extend previous understandings of multiplication to multiply a fraction or whole number by a fraction. 4.EE.17 – Perform operations with numbers expressed in scientific notation, including problems where both decimal and scientific notation are used. 4.R.FW.1 – Understand and use technology systems. 4.S.CC.4 – Demonstrate active listening skills.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI.2.1 – Participate in conversations, discussions, and written exchanges about a range of topics, texts, and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5 / HI.5.2 - Gather information from multiple print and digital sources ELP Anchor Standard 5/HI.5.7 - Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Percent - Review with students how to use the percent function on their calculators. - Ask students: How do we use percents in our everyday lives? Possible answers: - Sale prices - Home or car loans - Sales tax - Splitting a dinner bill with friends - Provide several examples on the board to allow students to practice performing percent calculations. Activity 2 Worksheet 4-1: Allowable Burr Size on Stampings1 Notes Project Worksheet 4-1: Allowable Burr Size on Stampings1 (page 34) on board. - Walk through the diagram, pointing out the area labeled BURR. - Explain that a burr can be small ridges or protrusions on a metal part that is a minor imperfection, caused by the punch going through the metal. - Small burrs are common but when they are too large, they cause problems. - The press operator making the part must measure it with a micrometer and decide if it is acceptable. - Rule of thumb: the height of the burr cannot be greater than 5% of the material thickness. - Ask students what would be the first thing you do when starting this job? Calculate the allowable burr. - Write on the board the following and have students calculate the maximum allowable burr height: 0.016” thick; 0.036” thick; 0.042” thick; 0.010” thick. Answers: 0.016 x 5% Or 0.016 x 0.05 = 0.0008; 0.036 x 5% Or 0.036 x 0.05 = 0.0018; 0.042 x 5% Or 0.042 x 0.05 = 0.0021; 0.010 x 5% Or 0.010 x 0.05 = 0.0005. - Review answers with students and check for understanding. Activity 2 Worksheet 4-2: Gage Maker’s Tolerance for a GO/NO-GO Gage1 Have students define: Gage maker (technician): Makes the tools an operator uses to measure the allowable upper and lower limit tolerances. GO/NO-GO Gauge: an inspection tool that measures a workpiece against its allowed tolerances. Some are a single tool with one end green (GO) and the other red (NO-GO), and some are individual tools with labels in green or red. It is a cost-effective way to verify that parts meet the drawing requirements. Write on the board: Æ.750 + 0.010 Æ1.056 + 0.0003 - 0.0000 - Remind students that Æ is the symbol for diameter. - Model the calculation of GO/NO-GO pins for the students using the Talk Aloud teaching strategy, displaying the calculations on the board. - Calculate the lower limit (GO pin,) 0.750 – 0.010 = 0.65, which is the minimum size - Calculate the upper limit (NO-GO pin) 0.750 + 0.010 = 0.76, which is the maximum size. - Add the tolerance to itself, 0.010 + 0.010 = 0.020 and multiply this by 10% = 0.002. - Calculate the upper and lower limits of the G)/NO-GO pins with 0.002 - Lower limit (GO pin) 0.750 – 0.002 = 0.748 - Upper limit (NO-GO pin) 0.750 + 0.002 = 0.752 - Have students try calculating the GO/NO-GO pins for Æ1.056 + 0.0003 - 0.0000 - Check for understanding. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 4

Assessment Students will successfully master the use of the calculator to perform percent functions to calculate burr sizes and g maker’s tolerances. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Activity 2 Students may find the directions for this activity difficult to understand, as it changes the way tolerances are determined. Provide students with plenty of examples until they feel comfortable with the definitions and the calculations. Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #5 - Identify angles, triangles, and circles. Calculate measurements of angles, triangles, and circles.

Module Content Determining the dimensions of objects described in a blueprint or drawing.

Targeted ABE/ASE Content Standards 3.MD.10 - Recognize Volume as an attribute of solid figures and understand concepts of volume measurement. 4.G.5 - Solve problems involving scale drawings of geometric figures, including computing actual lengths and areas from a scale drawing, and reproducing a scale drawing at a different scale. 4.G.17 - Apply the Pythagorean Theorem to determine unknown side lengths in right triangles in real world and mathematical problems in two and three dimensions. 4.R.RS.3 - Follow precisely a multi-step procedure when carrying out experiments, taking measurements, or performing technical tasks. 4.R.VA.3 - Demonstrate use of content, technical concepts, and vocabulary when analyzing information and following directions. 5.G.GMD.2 - Use volume formulas for cylinders, pyramids, cones, and spheres to solve problems. 4.R.FW.1 – Understand and use technology systems. 4.S.CC.4 – Demonstrate active listening skills.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions and written exchanges about a range of topics, texts and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5 / HI.5.2 - Gather information from multiple print and digital sources ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Angles1 Notes - Discuss with students how blueprints do not always provide every dimension of the object to be made. They will need to determine the dimensions from the drawing using basic geometry. If they are working in CNC shop, as either an operator or inspector, they will need to read drawings and specifications for the parts to be made. Shops like this use GD & T (geometric dimensioning and tolerancing) to make measurements. - Provide students with examples of tools used to measure angles and triangles such as: scales (rulers), protractors, squares, micrometers, verniers, dial and digital calipers - Ask students if they have used such tools before and how they used them. - Provide students with Learn It: Angles1 (pages 39-44), Unit 5. - Have students work in groups to read and discuss the notes provided. - Give each group of students protractors and several examples of angles to measure (geometric shapes or paper). - Visit the groups to help correctly measure and answer questions. Activity 2 Worksheet 5-1: Complementary and Supplementary Angles1 - Provide students with Worksheet 5-1: Complementary and Supplementary Angles1 (page 45) - Discuss angles, illustrating on the board the seven facts about angles, having the students read along as you model. - Model for students how to determine a complementary and supplementary angle, walking through the examples on the worksheet. - Ask students to find complementary and supplementary angles in the classroom. - Have them explain their findings and then have them measure the angles. - Examples (You will need to set these up before the activity): odd-shaped tables, partially open doors, a meter stick stacked across two parallel meter sticks, desk chairs Activity 3 Learn It: Angles in Triangles Notes & Worksheet - Provide students with Learn It: Angles in Triangles1 (pages 46-47) and Worksheet 5-2: Finding Angles on Prints (pages 48-49). - Discuss triangles, illustrating on the board the several types of triangles, and demonstrate how to prove that interior angles add up to 1800. - Have students note that angles on a blueprint cannot always be measured directly. Using the angle facts they learned will be necessary to determine those measurements. - Project page 48 and 49 on the board. - Model for students how to calculate the missing angle measurement from the measurements already given for the first example. - Have students calculate the remaining missing angles on pages 48 & 49, reviewing together as a class. - Provide students with protractors and calculate angles found in the classroom. Check for understanding. Activity 4 Measurement Practice – Cones & Cylinders Distribute Measurement Mania: Cones & Cylinders2 worksheet to all students. The instructor will review and model the use of the formulas in the worksheet with the measurement tools for this activity. Students will be divided into groups of 2-3, depending on class size and materials available. Each student will measure each object at their station, with each student confirming the measurements taken with the group members. Students should use various tools to measure each object, deciding on the one tool they feel gives them the most accurate measurement. Students will work together to calculate the volume, surface area, and perimeter of the measured objects. The instructor will review the answers with students and encourage students to make flash cards of the formulas for future reference and review. Measurements that are close should be considered correct; however, students should consider the tool they used and ask if they could get a more accurate measurement with another tool. Activity 5 Measurement & Practice – Tubes and Geometric Shapes Distribute Measurement Mania: Tubes & Geometric Shapes2 worksheet to all students. The instructor will review and model the formulas in the worksheet with the appropriate measurement tools. Students will view the video “The Difference Between Pipe and Tube” and complete the guided notes. https://youtu.be/RItW-vgFJUE?si=MTk2QHLxbQnyAImu The instructor will review the video and notes with students. After viewing the video, students will be divided into groups of 2-3, depending on class size and materials available. Each student will measure each object at their station, with each student confirming the measurements taken with the group members. Students should use various tools to measure each object, deciding on the one tool they feel gives them the most accurate measurement. Students will work together to calculate volume and surface area of the measured objects. The instructor will review the answers with students and encourage students to make flash cards of the formulas for future reference and review. Measurements that are close should be considered correct; however, students should consider the tool they used and ask if they could get a more accurate measurement with another tool. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 5 2 Prepared by D. Karavites-Uhl 2023

Assessment Students will demonstrate accuracy and precision in measuring and calculating angles, triangles, volume, area, and total surface area. Students will demonstrate the proper use of measuring tools (Vernier Caliper, dial caliper, micrometer, and rulers). Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Turn on closed captioning when showing the video. Allow students the opportunity to pause and rewind sections of the video as needed to improve comprehension. Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #6 - Identify and use formulas from industry handbooks and translate different units of measurement.

Module Content Finding solutions to problems in many companies is done through teams. Workers collaborating to find a solution often perform better at their job. Often the teams will need information beyond what they know. This is where reference manuals and industry specific texts become important.

Targeted ABE/ASE Content Standards 3.NF.13: Solve real word problems involving multiplication of fractions and mixed numbers (e.g., by using visual fraction models or equations to represent the problem). 3.NF.14: Apply and extend previous understanding s of division to divide unit fractions by whole numbers and whole numbers by unit fractions. 4.RP.1: Understand the concept of a ratio and use ratio language to describe a ratio relationship between two quantities. 4.NS.3: Fluently add, subtract, multiply, and divide multi-digit decimals using the standard algorithm for each operation. 4.NS.10: Apply and extend previous understandings of multiplication to multiply a fraction or whole number by a fraction. 4.EE.17 – Perform operations with numbers expressed in scientific notation, including problems where both decimal and scientific notation are used. 4.R.RS.3 - Follow precisely a multi-step procedure when carrying out experiments, taking measurements, or performing technical tasks. 4.R.VA.3 - Demonstrate use of content, technical concepts, and vocabulary when analyzing information and following directions. 4.R.FW.1 – Understand and use technology systems. 4.S.CC.4 – Demonstrate active listening skills.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions and written exchanges about a range of topics, texts and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5/HI.5.2 – Gather information from multiple print and digital sources. ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources Working in a team requires communication skills as well as math skills. Discuss with students what they know about communicating with others on the job. Examples of industry specific manuals would be useful for students to review during this outcome. All materials can be found in the Math Activities & Resources document. Activity 1 Calculating Tap Drill Size and Drill Clearance Provide students with Worksheet 6-1: Calculating Tap Drill Size1and Worksheet 6-2: calculate Drill Clearance1 (pages 55 & 56). Worksheet 6-1 - Review the formula used in solving the following problems: Tap Drill Size: TDS=Diameter – 1N and how to substitute in the correct numbers for each of the five steps. - Make note that step five should have the decimal converted to a fraction, and that is the drill size needed. - Remind students of PEMDAS - Example: Modeling Sample Problem #1 - Write on the board: - TDS = Diameter -1/N - Diameter is 3/8 Tap is 16 - 3/8 – 1/16 Convert these to decimals - 0.375 -0.0625 = 0.3125 - Convert 0.3125 to a fraction - 5/16 is the drill - Have students collaborate with a partner to complete the practice problems and review answers, checking for understanding. Worksheet 6-2 - Review the formula for drill clearance: DC = DIA/3 + Depth and how to substitute in the correct numbers for each of the five steps. - Remind Students of PEMDAS - Example: Modeling Practice Problem #3 - DC = DIA/3 = Depth - Diameter is ¼ Convert this to a decimal 0.0250 - Convert depth 1 ½ to decimal 1.500 - Divide 0.0250 by 3 and add 1.500 - 0.083 + 1.500 = 1.583 which is the drill clearance. - Have students collaborate with a partner to complete the practice problems and review answers. Activity 2 Calculating Drill Speeds Understanding the correct RPM (revolutions per minute) is important so as not to place stress on the tool and the machine. This could result in inferior quality, worn or broken tools, and result in a safety hazard. 1. Provide students with Worksheet 6-3: calculating Drill Speeds in Revolutions Per Minute (RPM)1(page 57). - Review the two main factors in determining drill speed: - What materials are you cutting? - What is the diameter of the drill? - Explain the harder the material, the lower the RPM; the larger the drill, the lower the RPM. With softer materials and smaller diameter drills, the RPM can be increased. - Review the formula: RPM = 4 x CS/DIA - Point out the Materials Chart they will need to use to complete the practice problems. - Remind students of PEMDAS - Example: Model Sample #5 - RPM = 4 CS/DIA - CS (cutting speed) = 60 and the drill is 5/16 convert 5/16 to a decimal = 0.312 - 4(60)/0.312 - 240/0.312 - RPM is 769 - Have students collaborate with a partner to complete the practice problems and review answers, checking for understanding. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 6

Assessment Students will demonstrate the use of formulas to solve decimal, fraction problems in contextualized formats and identify how these skills can be applied in industrial and business contexts. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #7 - Identify and calculate algebraic formulas to calculate simple interest due on a loan, compound interest on an investment, and use Cartesian coordinates to plot number pairs.

Module Content Math is more than adding, subtracting, multiplying, and dividing. To be successful, students will need additional skills like problem solving, critical thinking, and estimation.

Targeted ABE/ASE Content Standards 4.R.CL.6 - Integrate information from texts, charts, and graphs to draw a conclusion. 4.R.RS.7 - Integrate quantitative or technical information expressed in words in a text with a version of that information expressed visually (e.g., in a flow chart, diagram, model graph, figure or table). 4.R.RS.11 – Transcribe and interpret information, data, and observations to apply information learned from reading to actual practice. 4.R.VA.3 - Demonstrate the use of content, technical concepts, and vocabulary when analyzing information and following directions. 4.RP.5 - Recognize and represent proportional relationships between quantities. 4.F.4 - Construct a function to model a linear relationship between two quantities. 4.F.5 -Describe qualitatively the functional relationship between two quantities by analyzing a graph 4.R.FW.1 – Understand and use technology systems. 4.S.CC.4 – Demonstrate active listening skills.

Targeted ESL Content Standards ELP Anchor Standard 1 / HI.1.3 - Cite specific details and evidence from texts to support the analysis. ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions and written exchanges about a range of topics, texts and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5/HI.5.2 – Gather information from multiple print and digital sources. ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Simple and Compound Interest Discuss with students how they have used formulas in the past lessons to solve problems. Review the terms ‘variable’ and discuss how when performing the same calculation several times, a formula can be used to save time and effort. - Ask students why they might need to take out a loan for their business. Mortgage Truck payments Equipment and Supply purchases - Explain that monthly payments are based on the amount of money borrowed, the interest rate, how long you have to pay it back, and the number of payments. - Calculating the payments for a simple or compound interest loan is algebra. - Write the formula for simple interest on the board: - I = prt Interest = Principal x Rate x Time - I = interest - p = principal/original amount borrowed, - r = rate (convert % to decimal) - t = time in years to pay back the loan - Model the following scenario. - Frank borrowed $40,000 to purchase mill stock for his company, at a rate of 8.9% for 24 months. How much interest will he pay for 24 months and what will his payment be each month? - Model how to plug the numbers into the formula for simple interest - $40,000.00 x 8.9% (convert to decimal = 0.089) x 2 (24 months convert to number of years) - $7120.00 is the total amount of interest he will pay. - $47, 120. 00 is the total he will pay back to the bank. - The loan is for 24 months (2 years), his monthly payment will be 47,120.00/24 = $1963.33. - Frank wants to invest money at a local bank and purchases a CD (certificate of deposit) for $4,000 at a rate of 3% for 4 years. How much money will he earn with his investment? - Write the formula for compound interest on the board: - A = P (1 + r) t - A is the total amount to be paid monthly - P = principal - R = rate - T = time in years - Model how to plug the numbers into the formula on the calculator for compound interest, converting the percent to a decimal. - A = 4000.00 x (1 + 0.03)4 - A = 4502.04 (review how to round if needed) Activity 2 Cartesian Coordinates Cartesian coordinates can be used to pinpoint a location on a map or graph and are used in engineering to solve problems like finding the intersection of lines or curves, calculating distance, and determining the area of a region. Provide students with Work It Out: The Cartesian Plane and Coordinates1 worksheet and review the following ideas: - Machinists use coordinates to control CNC machines and other tools that cut and shape materials. - CNC machines use a Cartesian coordinate system based on an X, Y, and Z axis. - This allows the machine to move in a specific direction along a specific plane. - The “x” axis allows movement left and right - The “y” axis allows movement forward and backward - The “z axis allows movement up and down - The CNC machine can transform a block of material into a part with specific dimensions. - Show the following video: https://youtu.be/d99B8UmP4tE Project on the board/overhead the worksheet. - Model how to label the y and x axis, labeling each quadrant (1, 2, 3, & 4) and where the point of origin is located. - Label each quadrant with its positive and negative values (Diagram 1) Diagram 1 https://www.autodesk.com/products/fusion-360/blog/cnc-coordinate-system-made-easy/#:~:text=Nearly%20all%20CNC%20machine%20uses,Y%2C%20and%20Z%2Daxis. - Explain that coordinates are written in the form of (x, y) and have students follow along as you graph the first set of coordinates from the worksheet. - Have students complete the worksheet and check students’ answers. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 7 https://www.teachengineering.org/lessons/view/van_linear_eqn_less2

Assessment Students will exhibit mastery in calculating simple and compound interest, constructing and plotting coordinates on a four-quadrant graph, and identify the use of these concepts in manufacturing settings. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card formula flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Turn on closed captioning when showing the video. Allow students the opportunity to pause and rewind sections of the video as needed to improve comprehension. Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.

Outcome #8 - Understand and define mean, median, mode, and standard deviations.

Module Content Statistics is a branch of mathematics that draws inferences based on information contained in a sample. Manufacturing uses statistical process control (SPC) to check the process of making good parts. Not every piece can be checked, so checking how the parts are made (monitoring the process) is important for accuracy.

Targeted ABE/ASE Content Standards 4.SP.1 Recognize a statistical question as one that anticipates variability in the data related to the question and accounts for it in the answers. 4.SP.2 Understand that a set of data collected to answer a statistical question has a distribution which can be described by its center, spread, and overall shape. 4.SP.3 Recognize that a measure of center for a numerical data set summarizes all of its values with a single number, while a measure of variation describes how its values vary with a single number. 5.S.ID.3 Interpret differences in shape, center, and spread in the context of the data sets, accounting for possible effects of extreme data points (outliers). 5.S.IC.1 Understand statistics as a process for making inferences about a population parameter based on a random sample from that population. 5. S.MD.2 Calculate the expected value of a random variable; interpret it as the mean of the probability distribution.

Targeted ESL Content Standards ELP Anchor Standard 2/ HI. 2.1 – Participate in conversations, discussions and written exchanges about a range of topics, texts and issues. ELP Anchor Standard 2/ HI.2.5 – Ask and answer questions to clarify ideas and information. ELP Anchor Standard 5/HI.5.2 – Gather information from multiple print and digital sources. ELP Anchor Standard 5/HI.5.7 Include illustrations, diagrams, or other graphics as appropriate. ELP Anchor Standard 7/ HI.7.2 – Use a wider range of complex general academic and content-specific words and phrases.

Targeted IL IEL/CE Competencies EM11. Read safety signs and instructions commonly found at work. EM14. Describe ways employers may evaluate work performance.

Activities & Resources All materials can be found in the Math Activities & Resources document. Activity 1 Basic Statistics On the board write the following terms and symbols: Mean x̄; Median; ∑; Standard Deviation σ; Range. Average of a group of values; The actual or interpolated middle vale in a group of values; Total sum of all the values; How spread out the values are; The largest value minus the lowest value. Discuss with students the meaning of each term/symbol and have students take notes. Provide students with The Basic Math of Statistics1 worksheets (Pages 104-105). - Review Finding the Mean1 (page 104-105) with students, modeling the examples. - Check for understanding. Provide students with Worksheet 10-1: Mean vs. Median1 (106). - Have students collaborate with a partner to complete the worksheet and review the answers. - Discuss with students why it might be necessary to calculate the mean and median for a group of parts that was returned to the manufacturer. Activity 2 Standard Deviation Knowing the mean and median will give the manufacturer an idea of how well the process is working when making parts. However, it is also important to know the measurement variations from part to part. To determine the variation, a standard deviation calculation such as the “n-1” standard deviation is used. Provide students with Worksheet 10-2: Standard Deviation1 (pages 107-109). On the board write the equation for calculating standard deviation: SD n-1 = √ ∑ │x - x̄ │2 n - 1. Model the meaning of the equation using the example on pages 107-108, as students follow along, using the following guidelines: - Subtract the average from each score - Square each of these differences (multiply the difference by itself) - Add up all the squared differences (the sum of the squares) - Divided the squares by the number of games played - Take the square root of that number The calculation looks complicated, so assure students that the calculator will be doing the work. Use the Talk Aloud strategy for modeling each part of the equation: - Over ten games, the #2 guard averaged 19.2 pts per game. Mean = 19.2 - Subtract the mean (19.2) from each of the scores (example 21-19.2=1.8) - Square each of the answers (example 1.82 = 3.24) - Add up all the squared numbers (343.56) - Plug the numbers in to the equation at this point to show students the process: ∑ │x - x̄ │2 = 343.56 - There were ten games, so n= 10 and then n – 1 = 9. - Divide 343.56 by 9 = 38.1733. - Last step is to find the square root of 38.1733 SD n-1 = √ 38.1733 = 6.178 Check for understanding as students use their calculators with the example. Discuss with students that the standard deviation for this player’s score is more than 6 points. When compared to the mean of 19.2, it does not mean too much, as the other players will pick up the slack. In manufacturing, however, tolerances are set and if the tolerance is + 0.005, a standard deviation of 0.004 is not acceptable. If the total tolerance is 0.010, the standard deviation should be something like 0.001. Discuss with students why a low standard deviation is important when manufacturing parts for cars or medical equipment. Activity 2 Average and Standard Deviation Practice On the board write the following: Set 1; Set 2; Set 3. 1.205; 0.063; 0.701. 1.252; 0.061; 0.701. 1.255; 0.057; 0.712. 1.251; 0.058; 0.713. 1.255; 0.060; 0.710. Set; 1; 2; 3. Mean; 1.25626; 0.0598; 0.7074. Standard Deviation; 0.00068; 0; 0. Have students calculate the average of each set and the standard deviation. Activity 3 Range Using the same set of numbers from Activity 2, model how to find the range of each value set, subtracting the smallest value from the largest value. Set 1; Set 2; Set 3. 1.255 – 1.250 = 0.005; 0.062 - 0.057 = 0.005; 0.713 – 0.701 = 0.012. Provide students with Worksheet Exercise (page 111) for practice. Have students collaborate with a partner as you walk around checking for calculator accuracy and reviewing answers. 1 Math for Manufacturing Student Workbook, Ray Prendergast, Version 1.43, Unit 10

Assessment Students will demonstrate mastery in calculating mean, median and range, and standard deviation to determine the use of statistics in manufacturing quality parts. Students will complete and edit all worksheets, either through peer editing, instructor modeling, or the instructor answer key. Students will have completed index card flashcards and filed all worksheets in portfolio folder for review and future reference.

ELL Supports/Instructor Notes Reference the Manufacturing Bridge Introduction document for strategies to assist English Language Learners (ELLs). Consider pairing and grouping students of mixed math ability to increase opportunities for comprehension. Depending on the math knowledge students already have, some activities may need to be done through direct instruction rather than pair or group work.